ALTRO FORTIS TITANIUMTM 15

SECTION 10 26 00

WALL PROTECTION SYSTEMS

PART 1 	GENERAL

1.01	SECTION INCLUDES

- This section includes the following type of wall protection systems:

  - Wall Protection

1.02	RELATED SECTIONS

# A.	Section 10 26 00 section 10 26 00 “Wall and Door Protection”

1.	Wall Covering, Wall Panels, Corner Guards, Bumper Guards, Accent Rails, Crash Rails.

1.03	REFERENCES

# A.	General: Standards listed by reference, including revisions by issuing authority, form a part of this specification section to extent indicated. Standards listed are identified by issuing authority, authority abbreviation, designation number, title or other designation established by issuing authority. Standards subsequently referenced herein are referred to by issuing authority abbreviation and standard designation.

# B.	American Society for Testing & Materials (ASTM):

  - 1.	National codes (IBC, UBC, SBCCI, BOCA and Life Safety)

  - 2.	American Society for Testing and Material (ASTM)

  - 3.	National Fire Protections Association (NFPA 252 fire test)

  - 4.	Underwriters Laboratories (UL-10C positive pressure fire test)

  - 5.	CAN ULC Test Results

1.04	SYSTEM DESCRIPTION

- Performance Requirements: Provide Altro Fortis TitaniumTM 15 which has been manufactured by Altro to maintain performance criteria stated by manufacturer without defects, damage or failure.

1.05	SUBMITTALS

# A.	Product Data: Submit manufacturer’s current printed product literature, specifications, installation instructions, and field reports in accordance with Section 01330 - Submittal Procedures.

# B.	Shop Drawings: Submit shop drawings to indicate materials, details, and accessories in accordance with Section 01330 - Submittal Procedures including but limited to the following:

1.	Submit a layout diagram indicating the location of each panel and related installation details.

# C.	Samples: Submit duplicate sample pieces of Altro Fortis TitaniumTM 15 material in accordance with Section 01330 - Submittal Procedures.

# D.	Quality Assurance Submittals: Submit the following:

1.	Test Reports: Certified test reports showing compliance with specified performance characteristics and physical properties.

2.	Manufacturer’s Instructions: Current published manufacturer’s installation and maintenance instructions.

3.	Manufacturer’s Field Reports: Specified herein.

# E.	Closeout Submittals: Submit the following:

1.	Operation and Maintenance Data: Operation and maintenance data for installed products in accordance with Division 1 Closeout Submittals (Maintenance Data and Operation Data) Section. Include methods for maintaining installed products and precautions against cleaning materials and methods detrimental to finishes and performance.

2.	Warranty: Warranty documents specified herein.

1.06	QUALITY ASSURANCE

# A.	Installer Qualifications: Training: Installer who has a minimum of 3 years of experience installing wall protection products similar in complexity to those required for this project.

- Manufacturer’s Qualifications: Minimum 5 years of experience in the production of specified products and a record of successful in-service performance.

- Code compliance: wall protection products should conform to all applicable codes including IBC, UBC, SBCCI, BOCA and Life Safety.

- Chemical and stain resistance: Provide door protection products with chemical and stain resistance in accordance with ASTM D543.

- Provide wall protection products in accordance with the following:

1.	Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture and physical properties.

- Pre-installation Meeting: Conduct pre-installation meeting to verify project requirements, substrate conditions, manufacturer’s installation instructions and manufacturer’s warranty requirements.

# G.	Mock-ups: Install at project site a job mock-up using acceptable products and manufacturer approved installation methods. Obtain Owner’s and Consultant’s acceptance of finish color, texture and pattern, and workmanship standards.

  - Mock-Up Size: [Specify mock-up size.].

  - Maintenance: Maintain mock-up during construction for workmanship comparison; remove and legally dispose of mock-up when no longer required.

  - Incorporation: Mock-up may be incorporated into final construction upon Owner’s approval.

1.07	DELIVERY, STORAGE & HANDLING

# A.	Ordering: Comply with manufacturer’s ordering instructions and lead time requirements to avoid construction delays.

# B.	Deliver, store and handle Altro Fortis TitaniumTM 15 wall panels in accordance with Section 01610 - Basic Material Requirements.

# C.	Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

# D.	Store materials protected from exposure to harmful weather conditions, at temperature and humidity conditions recommended by manufacturer. Panels should be stored flat and be pre-conditioned a minimum of 72 hours in ambient temperatures similar to the prevailing operational conditions prior to installation.

# E.	HVAC must be up and running with ambient temperatures between 68F (20C) and 80F (27C) before, during and after installation a minimum of 72 hours.

# F.	When stacking pallets they should not be stacked more than 8 pallets high and stacked flat without bumps or rolls and in square and well aligned configuration, safety precautions should be taken to secure pallets to prevent shifting or falling.

# G.	Store adhesives, accessories and panels in temperature-controlled environments not subjected to freezing or excessive heat as this will compromise the integrity of the materials.  Leave protective blue film on panel until ready to use.

# H.	Wall panels, accessories and all other materials must be checked for defects and expiry before installation.  Do not install materials with visible defects or use expired adhesives and sealants.

- Installation areas must be enclosed and weather proofed before installation commences.  Verify other finishing operations, including painting, have been completed.

1.08	WASTE MANAGEMENT AND DISPOSAL

- Deposit all packaging materials in an appropriate container on site for recycling or reuse.

- Remove construction debris from project site and legally dispose of debris.

# C.	Avoid using landfill waste disposal procedures when recycling facilities are available.

# D.	Keep all discarded packaging away from children.

1.09	PROJECT CONDITIONS

# A.	Temperature Requirements: If storage temperature is below 68F (20C), the Altro Fortis TitaniumTM 15 panels must be moved to a warmer place and allowed to reach this temperature before installation.  For further information, refer to current Installation Guide.

- Maintain air temperature and structural base temperature at installation area between 68F (20C) and 80F (27C) for 72 hours before, during and after installation.

- Allow sufficient time for proper preparation, installation and curing. Verify other finishing operations, including painting, have been completed.

- Close spaces to traffic during installation until the installer is satisfied adhesive has set.

- Verify permanent HVAC is operational. If temporary heat is required, use electric or indirect heat sources. Do not use kerosene or propane in direct contact with the ambient air.

- If more than one pallet of material is used, use material in sequence.

- Wall panels and all other materials must be checked for defects and expiry before installation.  Do not install materials with visible defects or use expired adhesives and sealants. When in doubt, contact your local Altro Representative for assistance.

1.10	WARRANTY

# A.	Project Warranty: Refer to Conditions of the Contract for project warranty provisions.

# B.	Manufacturer’s Warranty: Submit, for Owner’s acceptance, manufacturer’s standard warranty document executed by authorized company official. Manufacturer’s warranty is in addition to, and not a limitation of, other rights Owner may have under Contract Documents.

# C.	Warranty Period for Altro Fortis TitaniumTM 15 shall be 20 years commencing on Date of Substantial Completion.  Please see current Altro Warranty online at www.altro.com Technical Documents.

1.11	EXTRA MATERIALS

# A.	Provide extra materials of product and adhesives in accordance with Section 01780 - Closeout Submittals.

# B.	Provide [______]sqft ([_______]m2) of each color, pattern and type material required for project for maintenance use.

# C.	Provide [______]sqft ([_______]m2) of extra materials in one piece and from same production run as installed materials.

# D.	Clearly identify each wall panel and each container of adhesive.

# E.	Deliver to Consultant, upon completion of the work of this section and store where directed.

PART 2	 PRODUCTS

2.01	MANUFACTURERS

- Manufacturer: Altro

1.	USA: 80 Industrial Way, Wilmington MA  01887
Toll-free: 800.377.5597  Fax: 610.746.4325

E-mail: support@altro.com Web Site: www.altro.com.

2.02	WALL PROTECTION PANELS

- Altro Fortis TitaniumTM 15 is an extruded semi-rigid PVC homogeneous panel and does not contain plasticizers or fillers.

- Acceptable material: Altro Fortis TitaniumTM 15 (measurements and product weights given below are approximate): Thickness: 0.60" (1.5 mm); Panel Width: 4’ (1.22m) Panel Height: 9’10” (2.993m); Weight per Panel: 17.6 lbs (7.9 kg).

  - 1.5 mm thick, 4’ x 9’10” panel

  - COLOR:

# Color to be selected from current range found at www.altro.com

# W150/ (insert alpha-numeric color code) (insert color name)

2.03	ACCESSORIES

Including but not limited to:

A.	Prefabricated Internal corners:  6” x 6” x 48” (stocking) or 6” x 6” x 118” (this size is fabricated on demand) - A526/30/ (insert numeric color code)

# B.	Prefabricated External corners: 6 ” x 6” x 48” (stocking) or 6” x 6” x 118” (this size is fabricated on demand) – A539/30/ (insert numeric color code)

# C.	Optional Accessories:

# 1.	3” x 3” x 48” - corner protector stainless steel - A861/12

# D.	Acrylic Adhesive:  For porous, even substrates and dry, climate-controlled areas use:

# 1.	Altro W157, one-part, water-based, acrylic adhesive as recommended by manufacturer.

# E.	Spray Adhesive:

# 1.	AltroFix 75, California compliant contact adhesive spray. Do not use with unbacked, plasticized vinyl.

# 2.	Altro W147, water-based acrylic blend, spray adhesive for use in dry climate-controlled areas and for most absorbent and non-absorbent surfaces.

# F.	Sealant and Mastic Compounds and Tools:

### 1.	ColorFlex sealant fills the gap between abutting panels, corners and may be used on perimeter.

2.04	SOURCE QUALITY

A.	Source Quality: Obtain product from a single manufacturer.

PART 3	 EXECUTION

3.01	MANUFACTURER’S INSTRUCTIONS

## A.	Compliance: Comply with manufacturer’s product data, including product technical bulletins, product catalog, installation instructions and product label instructions for installation.

3.02	EXAMINATION

# A.	Site Verification of Conditions: Verify substrate conditions, which have been previously installed under other sections, are acceptable for product installation in accordance with manufacturer’s instructions.

3.03	SUBSTRATE PREPARATION

- Substrate should be smooth and level. High points must be removed and low points filled with filler intended for the substrate and environmental conditions.

- Surfaces must be permanently dry and free from all substances that may contribute to adhesive bond failure.

- Remove loose paint and conduct an adhesive bond test with paint.

- Exterior walls must be adequately damp-proofed and insulated.

- Dry wall substrates should be paint ready.

3.04	PREPARATION

- All surfaces must be smooth, clean, dry, flat and free from dust and cleaned prior to Altro Fortis TitaniumTM 15 installation. Any debris, loose paint, plaster and dust must be removed. The working environment must also be dust free. Failure to comply with these conditions will reduce the bond strength between the adhesive and substrate and may cause the Altro Fortis TitaniumTM 15 panels to de-bond.

- Very absorbent / porous substrates (particularly plaster finishes and unprimed sheetrock) must have a proprietary sealer e.g. PVA primer or similar applied to the surface a minimum of 12 hours prior to the installation.

- All electrical switches, power points, etc., should be in a first fix / installation state.  All electrical equipment should only be moved or altered by a qualified electrician.

- If fitting to door frames, these must be in place prior to installation of Altro Fortis TitaniumTM 15.

- Prior to installation, it is advisable to complete any painting which comes in contact with Altro Fortis TitaniumTM 15, as sealant used at junctions is non-paintable.

- Panels should be stored flat and be pre-conditioned a minimum of 72 hours in ambient temperatures similar to the prevailing operational conditions.

- The panels must be stored on a level flat surface off the ground (risk of condensation on the panels if stored on damp surfaces). Storage on uneven surfaces could cause the panels to distort prior to installation.

- First, check the substrate using a 6’ (2 m) level to ensure all substrates are flat, paying particular attention to the doors, walls, corners, window reveals, and door entrances. These need to be inspected to ensure they are free of any debris or irregularities which could prevent the panels laying flat to the substrate after the adhesive has been applied and the panel installed.

3.05	INSTALLATION

## A.	Install Altro Fortis TitaniumTM 15 in accordance with the current Altro Fortis TitaniumTM 15 Installation Guide.  All joints should be joined by approved methods as detailed in the installation guide. Failure to install Altro Fortis TitaniumTM 15 in accordance with recommended procedures will void the Altro Limited Product Warranty.

3.06	FIELD QUALITY REQUIREMENTS

# A.	Manufacturer’s Field Services: Upon Owner’s request, provide manufacturer’s field service consisting of product use recommendations and periodic site visits for inspection of product installation in accordance with manufacturer’s instructions.

### 1. Site Visits: [Specify number and duration of periodic site visits.].

3.07	CLEANING

Note: Once all panels and joints are installed, remove protective film and clean all surfaces with antistatic solution or antistatic wipes. This is required as the panel may have static build up and any dust in the atmosphere will adhere to the surface of the panel.

- Altro Fortis TitaniumTM 15 can be cleaned with a diluted soap/detergent solution, see current maintenance.

- When cleaning the Altro Fortis TitaniumTM 15 surface, we recommend the temperature of water does not exceed 140° F (60° C).

- Pressure washing is not recommended.

- To reduce the buildup of static, cleaning the panels with an anti-static solution is recommended.  Use a lint-free, non-woven wipe or a polyester knit wipe.  Both are extremely low in particle generation.

- Do not use cleansers containing abrasives or solvents.

END OF SECTION